# The Respiratory Protection Compliance Checklist

### A Practical Guide to Moving Beyond the "Checkbox" and 
Ensuring Real Workplace Safety

Is your respiratory protection program actually protecting your workers, or is it just compliant on paper? Use this self-assessment tool to walk your facility, talk to your team, and identify hidden gaps before they turn into OSHA citations or health hazards.

## 📋 Part 1: Program Foundations & Oversight

Before evaluating how respirators are used in the field, you must ensure the structural framework of your program is legally compliant and properly managed.

- [ ] Designated Program Coordinator: Has your company designated a qualified, trained Program Administrator to run and oversee the daily operations of the respiratory program? (This is a strict OSHA requirement—even a written plan needs a captain.)

- [ ] Proper Respirator & Cartridge Selection: Have you identified and evaluated the specific respiratory hazards in your workplace (chemical state, physical form, and exposure levels) to select only NIOSH-certified respirators and appropriate cartridges? (Remember: one size or cartridge type does not fit all hazards!)

## 📋 Part 2: Field-Testing Your Program Evaluation

Program evaluations shouldn’t just check off that training and fit tests occurred. A true evaluation requires getting out onto the floor to see how the equipment is used in real-time.

- [ ] Employee Feedback: Have you spoken directly with respirator users to ask how the equipment fits, how comfortable it is, and if they have any difficulty breathing while wearing it?

- [ ] Donning & Doffing Observations: Have you watched employees put on and take off their respirators?

Real-World Warning: During a recent Job Safety Analysis (JSA), we spotted an employee wearing a full-face respirator that wasn’t remotely centered on the crown of their head! Poor fit means poor protection.

- [ ] Seal Checks: Are employees consistently performing positive and negative pressure user seal checks every single time they put on their respirator?

- [ ] Storage and Condition Checks: Are respirators being stored in clean, sealed containers away from dust and sunlight (not hanging from open hooks in work areas)?

- [ ] Scheduled Audits: Have you added respirator storage, physical condition, and cartridge changeouts to your facility’s monthly safety audit list?

## 📋 Part 3: Cartridge Changeout & Exposure Tracking

Cartridge life isn't a guessing game. It must be based on the specific hazard, the type of work being done, and the physical environment.

- [ ] Industrial Hygiene (IH) Monitoring: Have you conducted recent air monitoring to determine the actual airborne exposure levels in your facility? (Without this data, you cannot accurately select cartridges or determine lifespan.)

- [ ] Documented Changeout Schedule: Do you have a written plan that clearly defines when cartridges must be discarded and replaced?

- [ ] Employee Tracking Method: Is there a clear, simple method (like a log sheet or tag system) for employees to track how long their cartridges have been in use?

## 📋 Part 4: Respiratory Protection Training

Training should be tailored to your specific workplace hazards and respirator types. While initial training must cover everything, annual training can often be delivered efficiently 1-on-1 during scheduled fit testing.

Does your training program actively cover:

- [ ] Clear roles and responsibilities (both the company's and the user’s)?

- [ ] The specific respiratory hazards in your facility and the purpose of the program?

- [ ] Why specific respirator models were selected?

- [ ] The potential health risks of exposure and the documented exposure levels?

- [ ] Cartridge selection and the facility’s changeout schedule?

- [ ] Hands-on practice, including proper cleaning, maintenance, storage, and user seal checks?

- [ ] Fit-testing and medical clearance requirements?

## 📋 Part 5: Medical Clearances

Before an employee ever puts on a respirator or performs a fit test, they must be medically cleared. This is a critical legal and physical safeguard.

- [ ] LHCP Approval: Are all medical clearances evaluated and signed off by a licensed physician or other licensed healthcare professional (PLHCP/LHCP)?

- [ ] Baseline Questionnaire: Have all new employees completed the mandatory OSHA medical evaluation questionnaire before their first fit-test?

- [ ] Trigger-Based Updates: Do you have a process to trigger medical re-evaluations when changes occur?

Note: While periodic updates are often scheduled at regular intervals (annually to every 2 years), updates must also be triggered by specific events—such as an employee reporting medical symptoms, a change in workplace exposure levels, or physical changes like significant weight loss or dental work that affects respirator fit.

## 📋 Part 6: Fit Testing

- [ ] Compliant Fit Testing: Are all employees who wear tight-fitting respirators fit-tested initially and at least annually thereafter?

  - Are you using the right method?

    - Qualitative Fit Testing (QLFT): A pass/fail test relying on taste/smell, acceptable only for half-mask or filtering facepiece respirators (where protection levels are up to 10x the limit).

    - Quantitative Fit Testing (QNFT): An objective, machine-measured test, strictly required for full-facepiece respirators where higher levels of protection (up to 50x the limit) are needed.

### How Did Your Program Score?

If you checked every box, congratulations—your respiratory program is in excellent shape! If you found a few blank boxes, you aren't alone. These gaps are highly common, but easily fixable.

Need help setting up a cartridge changeout schedule, conducting IH monitoring, or performing a formal program audit?

Let’s chat. Contact us at (919) 676-2877 ext, 112 or through Contact Us to speak with a health and safety consultant today.

Three Essential Dimensions to Cultivate Organizational Well-Being –
 A Safe and Sound Workplace

#### Create a Safe Environment      	 Focus on Well-Being                   Cultivate Leadership Qualities